SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – Short Answers

| Course Code: | ITA03 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 2 – Short Answers |
| Weightage: | 30 % of CIA | Total Marks: | 100 Marks |
| CO1 Statement: | Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | Dhuneshwaran V | Reg. No.: | 192324169 |
| Section / Batch: | 2023 | Date: | 12/10/2026 |

Code of Conduct I ___Dhuneshwaran V / 192324169_____ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student: _________________________

Instructions - This test contains 50 questions, Total: 100 Marks. - Duration: 1 hour.

| Faculty In-charge | Course Coordinator | Head of Department |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

Assessment Tool 2 – Short Answers (CO1)

Course Outcome 1: Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems.

### Technical Report: Free-Space Path Loss Calculation (Question 17)

Problem Statement: At 900 MHz, at what distance does the free-space path loss reach 120 dB?

### 1. Introduction and Theoretical Framework

In wireless communication and cellular network design, evaluating signal propagation attenuation is critical for cell planning and coverage estimation. Free-Space Path Loss (FSPL) quantifies the signal strength reduction of a radio wave travelling through unobstructed free space (line-of-sight propagation) between an isotropic transmitter and receiver.

The generalized Friis transmission equation governing FSPL in terms of frequency and distance is:

FSPL (dB) = 20 log₁₀(d) + 20 log₁₀(f) + 20 log₁₀(4π / c)

For practical cellular engineering applications, substituting standard constants yields the standard empirical formula where frequency f is expressed in Megahertz (MHz) and distance d in Kilometers (km):

FSPL (dB) = 32.44 + 20 log₁₀(fMHz) + 20 log₁₀(dkm)

### 2. Mathematical Derivation and Step-by-Step Solution

Given parameters for the calculation:

- Target Free-Space Path Loss (FSPL) = 120 dB

- Carrier Frequency (f) = 900 MHz

- Substitute the known variables into the practical FSPL formula:
120 = 32.44 + 20 log₁₀(900) + 20 log₁₀(d)

- Compute the logarithmic value of the carrier frequency:
log₁₀(900) ≈ 2.95424
20 × 2.95424 ≈ 59.085 dB

- Aggregate the constant term and frequency contribution:
32.44 + 59.085 = 91.525 dB

- Substitute back into the equation:
120 = 91.525 + 20 log₁₀(d)

- Isolate the logarithmic distance term:
20 log₁₀(d) = 120 - 91.525 = 28.475

- Divide by 20 to isolate log₁₀(d):
log₁₀(d) = 28.475 / 20 = 1.42375

- Apply inverse logarithm (base 10) to solve for distance d:
d = 101.42375 ≈ 26.53 kilometers

### 3. Summary Table & Engineering Insights

| Parameter | Engineering Value |
| --- | --- |
| Operating Carrier Frequency (f) | 900 MHz |
| Target Path Loss (FSPL) | 120 dB |
| Calculated Propagation Distance (d) | 26.53 km |

Cellular Network Assessment: At 900 MHz—a standard frequency band for GSM and wide-area cellular deployments—a free-space path loss of 120 dB is achieved at a radius of approximately 26.53 km. In practical terrestrial deployments (such as urban or suburban environments), obstacles, multipath fading, and terrain diffraction cause actual path loss to accumulate much faster than ideal free-space conditions, resulting in significantly smaller real-world cell coverage radii for the same attenuation threshold.